# EGE322 — Assignment 2: BME280 Web Server

Module: EGE322 IoT System Project

Name: Nicholas Dubs

Student ID: [fill in if required]

## Submission checklist (zip: Assignment_2/)

Part 1/ — main.py, BME280.py

Part 2/ — main.py, boot.py

Part 3/ — main.py, boot.py

Part 4/ — main.py, boot.py

This Word document

## Part 2 — BME280 sensor web server (HTML table)

Screenshot of the web page showing temperature, humidity, and pressure in a table:

[INSERT SCREENSHOT: Part 2 — browser at ESP32 IP, sensor table visible, auto-refresh working]

Modified sections in main.py (compared to portal reference bme280_WebServer_1.py):

Yellow highlight = changed or added vs the original provided code.

def web_page():

bme = BME280.BME280(i2c=i2c)

temp = bme.temperature

pres = bme.pressure

hum = bme.humidity

html = """<html>

<head>

<meta http-equiv="refresh" content="5">

<title>ESP with BME280</title>

<style>

table { margin: auto; border-collapse: collapse; ... }

thead th { background-color: #1a56b0; color: white; ... }

</style>

</head>

<body>

<h1>ESP with BME280</h1>

<table>

<thead>

<tr><th>MEASUREMENT</th><th>VALUE</th></tr>

</thead>

<tbody>

<tr><td>Temp. Celsius</td><td>{}</td></tr>

<tr><td>Pressure</td><td>{}</td></tr>

<tr><td>Humidity</td><td>{}</td></tr>

</tbody>

</table>

</body>

</html>""".format(temp, pres, hum)

return html

Summary of changes:

Replaced three <p> tags with an HTML <table> (thead + tbody) as required.

Added CSS for table header, alternating row colours, and centred layout.

Read sensor values into variables before .format() (same data, clearer structure).

Uses BME280 library module uploaded to the ESP32.

## Part 3 — Two LEDs with ON/OFF web buttons

[INSERT SCREENSHOT: Part 3 — web page showing LED1 and LED2 states with ON/OFF buttons]

Modified sections in main.py (compared to portal reference web_server_led_on-off.py):

led1 = Pin(2, Pin.OUT)

led2 = Pin(4, Pin.OUT)

def web_page():

led1_state = "ON" if led1.value() == 1 else "OFF"

led2_state = "ON" if led2.value() == 1 else "OFF"

...

<p>LED1 state: <strong>{}</strong></p>

<p><a href="/?led1=on"><button ...>ON</button></a></p>

<p><a href="/?led1=off"><button ...>OFF</button></a></p>

<hr>

<p>LED2 state: <strong>{}</strong></p>

<p><a href="/?led2=on">...</p>

<p><a href="/?led2=off">...</p>

if '/?led1=on' in request:

led1.value(1)

if '/?led1=off' in request:

led1.value(0)

if '/?led2=on' in request:

led2.value(1)

if '/?led2=off' in request:

led2.value(0)

Summary of changes:

Added second LED on GPIO 4 (LED1 on GPIO 2, LED2 on GPIO 4).

Duplicated ON/OFF button pairs — one set per LED.

Changed URL query from /?led=on|off to /?led1=on|off and /?led2=on|off.

Request handler updated to control led1 and led2 independently.

## Part 4 — Combined sensor + LED control page

[INSERT SCREENSHOT: Part 4 — final combined dashboard (sensor readings + both LED controls on one page)]

Part 4 merges Part 2 sensor table/grid and Part 3 dual-LED controls into a single web_page(), with improved card-style CSS and 5-second auto-refresh for live sensor data.