# EGE322 Assignment 3 Jigsaw Learning

# Thingsboard Exploration Guide

# Introduction

## Purpose

This is a guide to setup and connect and ESP32 to ThingsBoard Cloud using MQTT, publish JSON telemetry, and to build a live dashboard. Any reader with access to an ESP32, WiFi, and relevant programming tools should be able to complete the task without extra help

## Scope

| Included | Not Included |
| --- | --- |
| ThingsBoard account, device, access token | ThingSpeak Partner Task |
| Dashboard Widgets | Adafruit IO (Separate, covered in Lab 8 |
| ESP32 main.py + boot.py | Setup of device control and programming |

This example uses internally generated sample data, with no extra sensor data. The guide uses a counter counting from 1 to 100 in steps of 10 every 2 seconds. Optional simulated temperature and humidity values can be sent when running the PC script in “rich” mode (more on that later) if a DHT22 is already wired as in Lab 7, see Appendix A

## Architecture

# ThingsBoard Account Setup

## Create a cloud account

- Open https://thingsboard.cloud and sign up (free tier is enough)

- Log in to tenant dashboard

Figure 1: Tenant Dashboard

## Create a device

- Click Add a device

- Name: ESP32-Counter

- Label (Opt): ESP32 Counter

- Leave the rest as-is

- Optional description

- Click: Next: Credentials

## Copy access token

- Open device ESP32-Counter

- Click Copy access token

- Keep for later. Will be used in main.py

- Do NOT share token publicly (treat it like a password)

- Click: Add

## MQTT connection settings

| Setting | Value |
| --- | --- |
| Broker host | Mqtt.thingsboard.clooud |
| Port | 1883 |
| Client ID | Any unique string (e.g. esp32_thingsboard) |
| Username | Device access token (from 2.3) |
| Password | Leave empty |
| Publish topic | v1/devices/me/telemetry |
| Payload format | JSON object with double quoted keys* |

*Example payload: {"counter": 10, "temperature": 24.5, "humidity": 55.0}
Invalid example: {temperature:24}

Test connection by running the following code in Windows command prompt (Replace   with token from 2.3:

curl -v -X POST http://thingsboard.cloud/api/v1/{YOUR_ACCESS_TOKEN}/telemetry --header Content-Type:application/json --data "{counter:10 ,temperature:24.5, humidity:55.0}"

# Build the Dashboard

## Create a dashboard

### Option 1: Generate Dashboard with AI

- Click: Generate Dashboard with AI

- Trust and hope that the dashboard looks semi decent

- Edit dashboard with the right charts and graphs

### Option 2: Manual Dashboard Creation

- Go to Dashboards

- Click + Add Dashboard -> Create new Dashboard

- Name: EGE322 Assignment 3

- Description (Opt): EGE322 Assignment 3 Telemetry Dashboard

- Click: Add

## Managing Widgets

- Click + Add Widget

- Click on Charts

- Click on Value and Charts Card

- Time window:

  - Realtime -> Last -> 10 minutes ->

  - Aggregation: None

- Datasource:

  - Device: ESP32-Counter

  - Data key: counter

- Go wild with the rest ig

# ESP32 setup and MicroPython code

## Prerequisites

| Item | Notes |
| --- | --- |
| Thonny | Connected to ESP32 |
| boot.py | WiFi SSID and password only — do not duplicate WiFi in main.py |
| Library | umqtt.robust on the board |
| Reboot | After editing files use Ctrl+D (soft reboot) so boot.py runs before main.py |

## Install code on the ESP32

- Open Thonny → ESP32 files.

- Confirm boot.py connects WiFi on boot.

- Create/replace main.py with the code in §4.4.

- Replace PASTE_DEVICE_ACCESS_TOKEN_HERE with your token (keep the b"..." bytes form).

- Save to ESP32 → soft reboot Ctrl+D.

[Figure 11 — Thonny showing main.py with token REDACTED]

## Run and verify

Expected shell output:

WiFi OK: 192.168.x.x

ThingsBoard MQTT connected

Published: {"counter": 0, "temperature": 24.0, "humidity": 50.0}

On ThingsBoard:

- Entities → Devices → ESP32-Counter → Latest telemetry

- Confirm keys counter, temperature, humidity update.

## Full MicroPython listing (assignment — internal sample data)

No external sensor required. Counter matches Basic Task A pattern; temperature and humidity are simulated for chart widgets.

## Code notes for troubleshooting:

| Line / idea | Why it matters |
| --- | --- |
| user=ACCESS_TOKEN | ThingsBoard uses the token as MQTT username |
| password=b"" | Password must be empty for device token auth |
| json.dumps(...) | Builds valid JSON; required for multi-key telemetry |
| qos=1 | At-least-once delivery; matches PC test script |
| wait_for_wifi() | Avoids MQTT connect before boot.py finishes |
| Counter 0→100 step 10 | Same pattern as ThingSpeak assignment sample |

# Results

With main.py running, the dashboard should show:

- Gauge: counter stepping 0, 10, 20, … 100, then resetting.

- Chart: three series updating every ~2 seconds.

(I NEED TO RUN THE CODE THEN TAKE SS OF THE DAHSBOARD)

Figure 14 caption: ThingsBoard dashboard EGE322 Assignment 3 receiving live MQTT telemetry from ESP32 device ESP32-Counter.

# Troubleshooting

| Problem | Likely cause | Fix |
| --- | --- | --- |
| Dashboard widgets empty | Wrong telemetry key | Set widget key to counter exactly |
| MQTT connect failed | Bad or expired token | Re-copy from device page |
| WiFi not connected | boot.py wrong / not run | Fix SSID/password; Ctrl+D reboot |
| OSError -202 after Run (F5) | boot.py skipped | Use Ctrl+D not F5 on main.py only |
| Chart flat, gauge works | Chart keys mismatch | Add temperature, humidity to JSON |
| Invalid JSON error | Manual string concat | Use json.dumps() |

# Limitations

- ThingsBoard free tier — limits on devices, messages, and retention; fine for coursework but not high-frequency production.

- Network — ESP32 needs 2.4 GHz WiFi; guest networks may block MQTT port 1883.

- Security — Access token grants full device access; redact in screenshots and do not commit to public Git.

- Latency — Cloud round-trip typically 1–5 s; not hard real-time control.

- Scope — This guide covers uplink telemetry only, not ThingsBoard rule chains or alarms.

- Platform choice — Adafruit IO (Lab 8) uses per-feed topics; ThingsBoard uses one JSON telemetry topic — different mental model.

# What I learned

ThingsBoard treats each ESP32 as a device with an access token used as the MQTT username. Telemetry is one JSON object per publish on v1/devices/me/telemetry, and dashboard widgets bind to keys inside that object. Compared to ThingSpeak MQTT (field1=value strings) and Adafruit IO (username/feeds/name per value), ThingsBoard feels closer to industry IoT platforms. Testing with a PC script before flashing the board saved time when tuning widget keys. Teaching this flow reinforced that documentation and screenshots matter as much as working code for peer learning.

# Appendix A — Optional: real DHT22 (Lab 7 wiring)

If you already wired DHT22 DATA → GPIO 15 from Lab 7, use the following code instead of §4.4. It publishes real temperature and humidity with the same counter pattern. Widget setup in §3 is unchanged.